Lab Example:

Given:	4 S/C distributed in two orbital planes

S/C in same plane are 180 deg apart from each other

mission success requires visibility of 2 S/C from operational ground site

Find:	develop a digital logic circuit that captures the conditions when the mission can/can’t be accomplished

Solve:	let “1” represent a visible S/C

“0” represents a S/C that isn’t visible

look for truth table scenarios with 2 visible S/C, BUT

some scenarios are impossible (can’t see 2 S/C in the same orbit plane)

- Complete the truth table

- Create a logic statement

- Simplify your logic statement

| 1st Plane |  | 2nd Plane |  | Mission Success |
| --- | --- | --- | --- | --- |
| A | B | C | D |  |
| 0 | 0 | 0 | 0 |  |
| 0 | 0 | 0 | 1 |  |
| 0 | 0 | 1 | 0 |  |