Lab #1 Debugging Guide

Problem: MCU worked previously but now not connecting or showing up in Arduino IDE.

- Check wiring thoroughly. Shorts (e.g. a resistor leg touching and LED leg) can cause MCU to not be recognized.

- Make sure correct MCU pins are utilized correctly (e.g. an analogRead() call on a non-ADC pin causes failure).

- If above does not fix problem:

  - Remove MCU from breadboard (so nothing is connected to it), do a boot/reset.

  - Restart Arduino IDE.

  - Try and upload a minimal sketch (either blink LED_BUILTIN or see Task 0C).

- If still not working, these problems are very rare, but worth trying:

  - Bad USB cable

  - Bad USB port

  - Bad MCU

  - Restart computer

RP2040 Debugging Checklist (Arduino / PlatformIO)

1. Power, USB, and Boot Mode (Highest Failure Rate)

- USB cable supports data

  - Many “charging-only” cables cause silent failures

- Power LED behavior

  - Verify the board actually powers when plugged in

- BOOTSEL state

  - BOOTSEL must NOT be held during normal operation

  - If BOOTSEL is held at reset, the board enters USB mass-storage mode and will not run sketches

- External power

  - VSYS and 3V3 pins used correctly

  - Do not back-drive 3.3 V rails unless the board explicitly supports it

2. Board Definition and Core Selection

- Correct RP2040 core installed

  - Arduino IDE: Earle Philhower “Raspberry Pi Pico/RP2040” core

  - PlatformIO: platform = raspberrypi

- Correct board selected

  - Pico ≠ Pico W ≠ Adafruit Feather RP2040

  - Wrong board often uploads successfully but runs incorrectly

- Core mismatch symptoms

  - Sketch uploads but never runs

  - USB serial never enumerates

  - GPIO mapping appears incorrect

3. Upload Process Verification

- Upload feedback

  - Confirm successful upload message

  - No UF2 copy errors

- Forced recovery

  - Hold BOOTSEL → plug in USB → board mounts as drive

  - Re-upload UF2 if the board appears “bricked”

- Auto-reset expectations

  - Some RP2040 boards do not auto-reset reliably

  - Manual reset after upload may be required

4. Minimal Sanity Test (Do Not Skip)

- Run Blink

  - Confirm correct LED pin

    - Pico: GPIO 25

    - Many boards: LED_BUILTIN mapped correctly, but verify

- Modify Blink delay

  - Confirms the code actually updated

- If Blink fails

  - Root cause is core / board / USB / boot mode, not logic

5. USB Serial Behavior (Critical RP2040 Difference)

- Serial startup timing

  - RP2040 USB serial enumerates after reset

  - Early prints may be lost

- Best practice:

- Serial.begin(115200);

- while (!Serial && millis() < 3000) {}

- Serial Monitor settings

  - Correct baud rate

  - Correct line endings

- Symptom: “Serial Monitor is blank”

  - Code is running, but USB not yet connected

6. Dual-Core Awareness (Subtle but Important)

- Default Arduino core

  - Runs on core 0 only

- If using setup1() / loop1()

  - Verify core-1 code is actually needed

  - Avoid shared variables without synchronization

- Common failure

  - Race conditions between cores

  - Shared globals corrupted

- Rule for students

  - Do not use core 1 until core 0 is fully working

7. GPIO and Pin Mapping Pitfalls

- GPIO numbers ≠ physical pin numbers

  - Always use GPIO numbering

- Verify pin capabilities

  - Not all pins support PWM, I2C, SPI

- Internal pull-ups

  - RP2040 pull-ups are weaker than AVR

  - External resistors often required

- Active-low signals

  - Buttons frequently read inverted

8. Analog I/O (Very Common Confusion)

- ADC specifics

  - 12-bit ADC (0–4095)

  - Input range 0–3.3 V only

- ADC pins are limited

  - Pico: GPIO 26–28 only

- No true DAC

  - analogWrite() is PWM, not analog voltage

- Floating ADC inputs

  - Will produce noisy or random values

9. Timing, Delays, and Performance

- RP2040 is much faster than AVR

  - Delay-based timing assumptions may break

- Use millis() / micros() carefully

  - Avoid blocking loops

- USB timing sensitivity

  - Long blocking code can stall USB serial

10. Libraries and Compatibility

- Library architecture support

  - Not all Arduino libraries support RP2040

- Symptoms of incompatibility

  - Compiles but behaves incorrectly

  - Uses AVR-specific registers or interrupts

- Validation

  - Run unmodified library examples

  - Check library README for RP2040 support

11. Memory and Stability Issues

- RAM is plentiful but not infinite

  - Large buffers can still crash programs

- Stack corruption signs

  - Random resets

  - Variables changing unexpectedly

- Watchdog behavior

  - Long blocking loops can trigger resets if enabled

12. Debugging Without Serial (Important Skill)

- LED-based debugging

  - Blink patterns indicate program state

- GPIO probing

  - Toggle a pin to confirm execution path

- External tools

  - Multimeter or logic analyzer for signal verification

13. Isolation Strategy (Engineer’s Method)

- Reduce to minimal sketch

- Disconnect all peripherals

- Test one pin or sensor at a time

- Rebuild incrementally

14. Required Pre-Help Checklist (Student Accountability)

Before asking for help, students must state:

- Board model and core version

- How the board is powered

- Whether BOOTSEL is involved

- Does Blink work? (Yes/No)

- Minimal sketch that reproduces the issue

- Observed behavior vs expected behavior

Instructor Notes

- Most RP2040 failures are USB/boot/board-definition issues, not logic.

- Enforce the Blink → Serial → Peripheral progression.

- Discourage immediate dual-core usage in introductory labs.

If you want next steps, I can:

- Compress this into a one-page RP2040 lab handout

- Create a grading rubric tied to this checklist

- Adapt it specifically for PlatformIO-only workflows

- Add a “known RP2040 gotchas” quick reference